/***********************************************************************

GPIB.CONF IEEE488 library config file

-------------------

copyright            : (C) 2002 by Frank Mori Hess

(C) 1994 by C.Schroeter

email                : fmhess@users.sourceforge.net

***************************************************************************/

/***************************************************************************

*

*   Syntax:

*

*         interface { ... } starts new interface board section

*         device {...} device configuration

*

***************************************************************************/

/* This section configures the configurable driver characteristics

* for an interface board, such as board address, and interrupt level.

* minor = 0 configures /dev/gpib0, minor = 1 configures /dev/gpib1, etc.

*/

interface {

minor = 0       /* board index, minor = 0 uses /dev/gpib0, minor = 1 uses /dev/gpib1, etc. */

board_type = "ni_usb_b" /* type of interface board being used */

name = "violet" /* optional name, allows you to get a board descriptor using ibfind() */

pad = 0 /* primary address of interface             */

sad = 0 /* secondary address of interface           */

timeout = T3s   /* timeout for commands */

eos = 0x0a      /* EOS Byte, 0xa is newline and 0xd is carriage return */

set-reos = yes  /* Terminate read if EOS */

set-bin = no    /* Compare EOS 8-bit */

set-xeos = no   /* Assert EOI whenever EOS byte is sent */

set-eot = yes   /* Assert EOI with last byte on writes */

/* settings for boards that lack plug-n-play capability */

base = 0        /* Base io ADDRESS                  */

irq  = 0        /* Interrupt request level */

dma  = 0        /* DMA channel (zero disables)      */

/* pci_bus and pci_slot can be used to distinguish two pci boards supported by the same driver */

/*      pci_bus = 0 */

/*      pci_slot = 7 */

master = yes    /* interface board is system controller */

}

/* Now the device sections define the device characteristics for each device.

* These are only used if you want to open the device using ibfind() (instead

* of ibdev() )

*/

device {

minor = 0       /* minor number for interface board this device is connected to */

name = "sme03"  /* device mnemonic */

pad = 28        /* The Primary Address */

sad = 0 /* Secondary Address */

eos = 0xa       /* EOS Byte */

set-reos = no /* Terminate read if EOS */

set-bin = no /* Compare EOS 8-bit */

}

device {

minor = 0

name = "scope"

pad = 8

sad = 0

}